BASICS OF BUCK COVERTER

- Lakshya Agrawal

A buck converter is a DC-to-DC power converter which steps down voltage from it’s input supply to the output and magnifies the current. The first thought that comes up is why not use a voltage divider instead? The answer is efficiency, while voltage dividers is consuming current all the time which increases losses a lot in the form of heat, buck converter works on a different principal, i.e. a switch mode which makes it much more efficient (around 90%). To know why a buck converter is more efficient, first we need to understand how is one made. A basic buck converter consists of five primary components arranged in the following manner:

- MOSFET(Q1): This is the ON-OFF Switch for the input voltage. Due to this the term Switched-Mode Power Supply originates. When looking for one, following properties should be considered:

  - Drain-to-source voltage rating: This is the maximum voltage the MOSFET can withstand.

  - Current rating: This is the maximum continuous current that can flow through the device.

  - Power dissipation rating: This describes how much power the MOSFET can safely convert into heat and release into its surroundings.

  - Thermal characteristics: This measures how difficult it is for heat to move out of the MOSFET. It is usually expressed in degrees Celsius per Watt

  - Duty cycle: It is the measure of how much time a signal is "active" (ON) compared to the total time of one complete cycle (ON + OFF).

- Inductor(L1): This is used in the circuit so there is no sudden spike of current in the circuit. It also stores magnetic energy. Following properties should be considered:

  - Inductance: Inductance is the property of an electrical conductor (usually a coil of wire) to oppose any change in the current flowing through it.

  - RMS Current Rating: Maximum RMS current that could flow without overheating the inductor

  - Saturation current rating: This is the amount of DC current flowing through the inductor that causes its inductance value to drop by a specific percentage (usually 10%, 20%, or 30%) from its initial value.

  - Equivalent series resistance: Value of resistance with which the inductor can be replaced and will produce identical heat losses.

- Output Capacitor (Co): It is required to help the inductor in maintaining constant voltage drop across the load.

  - Voltage rating: This is the maximum continuous DC voltage that can be applied to a capacitor without damaging the dielectric material.

  - Capacitance: Capacitance is the measure of a capacitor's ability to store an electrical charge per unit of voltage.

  - Equivalent series resistance: Value of resistance with which the inductor can be replaced and will produce identical heat losses.

  - Ripple current rating: This is the maximum RMS (Root Mean Square) alternating current that can flow through a capacitor continuously without causing it to overheat

- Input Capacitor (Ci): It is used to smooth out the voltage spikes that is caused by the very fast ON-OFF switching. This works both ways and remove any noise from the source so it does not disturb other electronics connected with the source supply.

- Rectifier Diode(D1): It provides an alternate path for the current to flow in the OFF state of MOSFET. It can be replaced by another MOSFET to create active/synchronous rectification.

  - Reverse voltage breakdown: The maximum voltage a diode can block when it is reverse-biased.

  - Forward voltage: The amount of voltage "lost" or dropped across the diode when it is conducting current.

  - Current rating: The maximum continuous current the diode can conduct in the forward direction without burning out.

  - Power dissipating rating: The total amount of heat the diode can safely release into the environment.

MODES OF OPERATION

The buck converter is a SMPS, that means it opens and closes in 10s of thousand of times in a second. This frequency decides how much current will be flowing through the buck converter. It has mainly two modes of operation:

- ON STATE: In this state, the circuit is closed and the current starts flowing through the inductor. The inductor showing its basic nature behaves as infinite resistance in the initial time and slowly starts behaving like an open circuit increasing the current linearly. During this time, it’s stored magnetic energy also starts increasing. The same amount of average current that flows through this inductor is also flowing through the load as they are in series.

- OFF STATE: In this state, the circuit is open and the stored magnetic energy comes into play as the diode provide an additional path to complete the circuit and the inductor starts supplying the current in the circuit by converting its magnetic energy following the basic principle of Lenz Law. Secondly, capacitors resist to change in voltage which help maintaining constant voltage drop across the load in this state.

INDUSTRIAL USE CASES

Consumer Electronics

This is the most common application. Most modern electronics run on low voltages (1.8V, 3.3V, or 5V), but their power sources are often much higher.

- The power supply unit (PSU) or battery provides around 12V to 19V. Buck converters on the motherboard step this down to the 0.8V–1.5V required by the CPU and GPU.

- Lithium-ion batteries provide about 3.7V–4.2V. Internal buck converters step this down to power the processor and memory modules efficiently to preserve battery life.

- Converting 12V (from a car cigarette lighter) or rectified AC mains down to the 5V standard for USB devices.

Automotive Industry

Modern vehicles are "computers on wheels" and require precise power regulation.

- Stepping down the 12V (car battery) or 24V (truck battery) to power dashboards, GPS, and touchscreens.

- While EVs have massive high-voltage batteries (400V–800V), the onboard electronics, lights, and sensors still operate on 12V. Large-scale buck converters manage this transition.

Industrial and Telecom Power Supplies

Industrial environments often use a "Distributed Power Architecture."

- Point-of-Load (POL) Regulation: In large server farms or telecom base stations, a main bus distributes 48V to minimize transmission losses. Individual buck converters are placed right next to the chips (the "Point of Load") to drop that 48V to the specific low voltage needed by the hardware.

- Control Systems: Powering PLCs (Programmable Logic Controllers) and industrial sensors that typically operate on 24V inputs but have internal 5V logic.

Renewable Energy Systems

Solar panels often produce varying voltages depending on sunlight intensity. A buck converter (often paired with MPPT technology) steps down the panel's high voltage to the specific level required to charge a 12V or 24V battery bank safely.

LED Lighting

LEDs are current-sensitive devices. Buck converters are used to provide a constant, stepped-down voltage from a higher DC source to drive LED arrays efficiently without the massive energy loss of a series resistor.

Exploring the Indian market

Asynchronous (non-synchronous) buck converters are staples in the Indian electronics market due to their simplicity, low cost, and robustness.

| IC/Module | Max Current | Input Voltage | Frequency | Notes |
| --- | --- | --- | --- | --- |
| LM2596 | 3.0A | 4.5V – 40V | 150 kHz | The "Industry Standard." Highly available as a cheap blue module. |
| XL4015 | 5.0A | 8.0V – 36V | 180 kHz | High current capability; often sold with a heat sink and CC/CV pots. |
| LM2576 | 3.0A | 4.0V – 40V | 52 kHz | Older technology, larger inductor required, but very reliable. |
| XL4016 | 8.0A – 10.0A | 4.0V – 40V | 180 kHz | Powerhouse for high-current DIY power supplies. |
| XL7015 | 0.8A | 5.0V – 80V | 150 kHz | Specialist for high-voltage inputs (e.g., e-bike batteries). |
| TPS5430 | 3.0A | 5.5V – 36V | 500 kHz | Higher frequency TI part; smaller footprint but slightly pricier. |